STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows for Workgroups v3.1 and v3.11 Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1996 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the driver files for Microsoft Windows

for Workgroups versions 3.1 and 3.11 from the SuperDisk to your

PC.

o  How to install the driver files for Microsoft Windows for

Workgroups versions 3.1 and 3.11.

o  Using Memory Managers.

COPYING THE DRIVER FILES

From the EZStart Driver Installation screen, select 'Copy Files' to

copy all of the files used by the Microsoft Windows for Workgroups

v3.1 or v3.11 driver from the SuperDisk to the Destination Directory

recommended by EZStart, or to a different directory that you specify.

The EZStart default Destination Directory is a:\NDIS.  Please make

a backup copy of your SuperDisk before EZStart copies files to the

SuperDisk to create an installation diskette.

INSTALLING THE DRIVER FILES

The following section describes how to install Windows for

Workgroups 3.1 or 3.11 along with the SMC driver. If you

already have Windows for Workgroups installed and only want

to install the SMC driver, proceed to "Installing the SMC Driver

Only".

Installing Windows for Workgroups and the SMC Driver

The following procedure describes how to install Windows for

Workgroups 3.1 or 3.11 along with the driver files. This procedure

is designed to expedite the driver installation. For information

on customizing your installation and Windows desktop, refer to

your Windows for Workgroups manual.

1. Insert the Windows for Workgroups Disk 1 Setup diskette in your

computer's disk drive.

2. Type: SETUP

3. Press the Enter key to start the setup procedure. Typically,

this procedure takes from 10 to 15 minutes.

4. Windows for Workgroups v3.1 users:

o  Follow the screen prompts until you receive a prompt that

says Windows has detected the SMC card and asks: "Do you want

to install the driver for this adapter now".

o  Select "No" since this driver is not as current as the one

that SMC provides.

o  When the "Install New Network Adapter" dialog box appears with

a list of adapters, select "Unlisted or Updated Network

Adapter".

o  Select "Continue".

o  Proceed to step 6.

5. Windows for Workgroups v3.11 users:

o  Follow the screen prompts until the Network Setup window

appears.

o  Select "Networks" and "Install Microsoft Windows Network".

o  Select "No additional network" or "Other", as necessary.

Refer to your Windows for Workgroups manual for information.

o  Select "Sharing" and specify whether you want other users to

be able to share your files and printer(s). Then select

"OK".

o  Select "Drivers".

o  Select "Remove" to remove the selected adapter driver since

this driver is not as current as the one that SMC provides.

o  When a message asks whether you are sure you want to remove

the driver, select "Yes".

o  Select "Add Adapter". The Add Network Adapter dialog box

appears.

o  Select "Unlisted or Updated Network Adapter" followed by

"OK".

o  Proceed to step 6.

6. When the "Install Driver" window appears, insert the SuperDisk

into your computer's disk drive.

7. In the "Install Driver" window, type the path where the

driver resides (for example, A:\NDIS).

8. Select "OK". The "Unlisted or Updated Network Adapter" window

appears.

9. Select the appropriate SMC adapter followed by "OK".

10. If you are prompted for the adapter's Base I/O setting,

enter it now.

11. Windows for Workgroups v3.1 users:

o  When the "Install Driver" window appears, insert the Windows

for Workgroups 3.1 Disk 8 diskette in your computer's disk

drive.

o  Verify that the drive shown in the window is the same drive

where you inserted the Windows for Workgroups 3.1 Disk 8

diskette, then select "OK".

o  During the following driver-installation procedure, you

will be prompted to insert a Windows for Workgroups

diskette or the SuperDisk. Insert the appropriate disk,

verify that the path name is correct, and press Enter.

o  Follow the screen prompts until you are prompted to restart

Windows or continue.

o  Proceed to step 12.

12. Windows for Workgroups v3.11 users:

o  When the Network Drivers window appears, you can use

"Add Adapter" and/or "Add Protocol" to add more adapters

(up to four) and protocols. Refer to your Windows for

Workgroups manual for more information.

o  To change the Base I/O setting you specified in step 10

and/or select a driver type, select "Setup" and make the

appropriate selections.

o  Select "Close" and "OK".

o  Follow the screen prompts to complete the driver

installation.

13. At the end of the installation, you will be prompted to

accept all the options you selected and restart

your computer or to return to DOS. Restart your computer so

your options can take effect.

Installing the SMC Driver Only

The following procedure describes how to install the SMC driver

and assumes that Windows for Workgroups 3.1 or 3.11 is already

installed.

Windows for Workgroups 3.1:

1.  Double-click on the "Control Panel" icon in the "Main" window.

2.  Double-click on the "Network" icon.

3.  Select the "Adapters" icon in the "Options" area.

4.  Select the adapter you want to remove followed by "Remove".

5.  When a message asks whether you are sure you want to remove

the adapter, select "Yes".

6.  Select "Add". The "Install Network Adapter" dialog box appears.

7.  Select the SMC adapter you want to install followed by "OK".

The "Install Driver" dialog box appears.

8.  Type the path where the driver resides (for example, A:\NDIS)

and select "OK". The "Unlisted or Updated Network Adapter"

dialog box appears.

9.  Select "OK" when the next two dialog boxes appear.

10. Select "Close". The "Network Settings" dialog box appears.

11. Select "OK". You will be prompted to accept all the options

you selected and restart your computer or to return to DOS.

Restart your computer so your options can take effect.

Windows for Workgroups 3.11:

1.  Double-click on the "Network Setup" icon in the "Network" window.

2.  When the "Network Setup" dialog box appears, refer to step 5

under "Installing Windows for Workgroups and the SMC Driver".

USING MEMORY MANAGERS

If you use Microsoft Windows 3.x or Windows for Workgroups, exclude

the adapter RAM window from the "386 Enhanced" area of the SYSTEM.INI

file. For example:

[386Enh]

EMMEXCLUDE=D000-D3FF

If you use a memory manager other than EMM386, refer to the manual

that came with it for information on how to exclude memory.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EliteView, EZStart, SuperDisk, and UltraChip

are trademarks of Standard Microsystems Corporation.  Other product

and company names are registered trademarks or trademarks of their

respective holders.